1. Damage Scenario
Typically, when the flash is damaged, even the U-Boot is corrupted. After powering on the board, there will be no output in the serial port terminal.

2.Install the Driver
Follow the steps in the driver installation document to install the necessary USB driver.

3.Connect the Board
Connect the board to power, then connect both the USB cable and the serial cable.The setup should look like this:

4.Navigate to the IPQ Directory
Open the directory containing the ipq recovery files.

5.Open Terminals
Open a Command Prompt window in the ipq directory and a serial terminal (e.g., PuTTY) to monitor logs.

6.Identify the COM Port
Check Device Manager to confirm the COM port number assigned to the USB serial device (in this example, it's COM4).

7.Execute Recovery Commands
In the Command Prompt window within the ipq directory, run the following two commands. Replace COM4 with your actual COM port number identified in the previous step.

.\QSaharaServer.exe -p \\.\COM4 -s 13:xbl.elf -v 3

.\QsaharaServer.exe -p \\.\COM4 -s 1:cdt-AP-AL02-C15_256M32_DDR4.bin -s 34:devcfg_noac.mbn -s 25:tz.mbn -s 23:rpm.mbn -s 5:openwrt-ipq9574-u-boot.mbn -s 35:apdp.mbn -s 37:tmel-ipq95xx-firmware.elf -v 3

After running the first command, the log output should resemble this:

After running the second command, the log output should resemble this. Upon successful recovery, the board will automatically reboot and enter U-Boot.